# LV Final Task – Report

GitHub Repository:

https://github.com/omarbeendeaded/LV-Final-Task/tree/final

GUI:

- Readings from Arduino are displayed on the left side.

- Input buttons on the right.

- Front lights toggle button.

- Flasher and horn are hold buttons.

Code:

- Secondary Arduino reads values from potentiometer, voltage sensor, ultrasonic sensor and sends them to primary Arduino.

- Primary Arduino reads data from lock button then transmits the first 4 pieces of data to LCD.

- Primary Arduino reads data from LCD and switches lights, flasher, and horn accordingly .

- Addresses of each text box or button:

| Text/Button | Address |
| --- | --- |
| Speed | 0x1000 |
| Battery Percentage | 0x1100 |
| Parking Sensor | 0x1200 |
| Car Lock | 0x1300 |
| Front Lights | 0x1400 |
| Flasher | 0x1500 |
| Horn | 0x1600 |

Schematic:

- Switches used to simulate inputs from LCD.

- Front lights and Flasher gate driver circuit:

|  | Optocoupler is used to isolate circuits. |
| --- | --- |